Schedule 11

GENERAL TECHNICAL REQUIREMENTS FOR

STAFF-OPERATED BANKNOTE HANDLING MACHINES

Customer-operated banknote handling machines sort banknotes into one of the categories specified below. Machines which do not check banknotes for fitness do not need to distinguish between categories B1 and B2, specified in Table 1.

Table 1

Sorting and handling of banknotes by staff-operated machines

| No. | Category | Properties | Treatment |
| --- | --- | --- | --- |
| А | (1) Objects not recognised as banknotes issued by the NBS | Not recognised as banknotes issued by the NBS because of any of the following: – wrong image or format, – folded corner(s) or missing part(s), – banknotes similar to banknotes issued by the NBS, – banknotes not issued by the NBS, – feeding or transportation error of the machine. | The machine separates objects and banknotes and hands them over to the operator for further action. (1) The operator separates these objects from banknotes not fully recognised as banknotes issued by the NBS and banknotes issued by the NBS suspected to be counterfeit (2) The operator separates these banknotes for further action and withdrawal (3) The operator separates these banknotes for further action and withdrawal |
|  | (2) Suspect counterfeit banknotes | Image and format recognised, but one or more authentication feature checked by the machine not detected or clearly out of tolerance. |  |
|  | (3) Banknotes issued by the NBS that are not clearly authenticated | Image and format recognised, but not all authentication features checked by the machine are recognised because of quality and/or tolerance deviations. In most cases unfit banknotes. |  |
| B1 | Banknotes issued by the NBS that are identified as genuine and fit | All authenticity and fitness checks carried out by the machine giving positive results. | Can be used for recirculation. |
| B2 | Banknotes issued by the NBS that are identified as genuine and unfit | All authenticity checks carried out by the machine giving positive results. At least one fitness criterion checked giving a negative result. | Cannot be used for recirculation. |

Category A(2) banknotes are handed over to the NBS in accordance with the decision on handling money suspected to be counterfeit.

Category A(3) banknotes are handed over to the NBS in accordance with the above decision.

Special requirements for sorting of banknotes processed by staff-operated machines

1.	A banknote processing machine (BPM) must sort banknotes into categories A, B1 and B2 as set out in Table 1. The machine must fulfil one of the following two requirements:

1)	have at least three dedicated output stackers for physical separation of banknotes into A, B1 and B2 categories in order to avoid the intervention of the machine operator;

2)	have two output stackers for physical separation of banknotes and fulfil the following requirements:

–	the authenticity and fitness checks are conducted in the same pass. In this pass, B1 category banknotes must be sorted into one stationary output stacker, whereas A and B2 categories into a separate stationary output stacker;

–	if a category A banknote is identified as being present in the second output stacker, the operator must re-run the banknote(s) from the second output stacker. In this second pass, banknotes suspected to be counterfeits must be separated from the category B2 banknotes by sorting the former into a dedicated output stacker.

2.	A banknote authentication machine (BAM) must physically sort banknotes into categories A and B, for which at least two dedicated output stackers are needed to avoid the intervention of the machine operator.

BAMs with only one dedicated output stacker may however classify and sort banknotes if the following requirements are fulfilled:

1)	Each time a category A banknote is processed, the machine must stop the processing immediately and keep the category A banknote in a position that avoids any physical contact with authenticated banknotes;

2)	The result of the authenticity check must be indicated for any single category A banknote on a display;

3)	The machine must check for the presence of a category A  banknote when it stops processing, and processing can only be resumed after the physical removal of the category A  banknote by the operator;

4)	For each stop of the processing mode no more than one category A banknote can be accessible to the operator.