Schedule 7

MINIMUM STANDARDS

FOR AUTOMATED FITNESS CHECKING OF BANKNOTES

This Schedule lays down minimum standards for automated banknote fitness checking by banknote handling machines.

In the course of the fitness checks, banknotes with any defect in respect of which a mandatory requirement has been defined as set out below are unfit.

The acceptable tolerance level for the fitness checks by banknote handling machines is 5%. This means that a maximum of 5% of the banknotes that do not meet the fitness criteria may be misclassified by the machines and sorted as fit.

Table 1
List of sorting criteria for automated fitness sorting

| No. | Defect | Definition |
| --- | --- | --- |
| 1. | Soil | General distribution of dirt across the entire banknote |
| 2. | Stain | Localised concentration of dirt |
| 3. | Graffiti | Added image or lettering written or marked in any manner on a banknote |
| 4. | De-inked note | Lack of ink on part or whole of the banknote, e.g. a washed banknote |
| 5. | Tear | Self-explanatory |
| 6. | Hole | Self-explanatory |
| 7. | Mutilation | Banknotes with part(s) missing, along at least one edge (in contrast to holes) |
| 8. | Repair | Parts of one or more banknotes joined together by tape or glue or other means |
| 9. | Crumples | Multiple random folds |
| 10. | Limpness | Structural deterioration resulting in a marked lack of stiffness |
| 11. | Fold | Self-explanatory |
| 12. | Folded corner | Self-explanatory |

Further information on sorting criteria

1. Soil

Soil increases the optical density of banknotes. The following table specifies the maximum density increase of limit samples compared to new banknotes that banknotes may exhibit to be classified as fit. Banknotes with optical density increase specified in Table 2 are limit samples of banknotes that may be sorted as fit.

Table 2

Optical density levels

| Denomination | Maximum density increase of limit sample compared to new banknote | Filter |
| --- | --- | --- |
| 10 dinars | 0.08 | Magenta |
| 20 dinars | 0.08 | Magenta |
| 50 dinars | 0.08 | Magenta |
| 100 dinars | 0.07 | Magenta |
| 200 dinars | 0.07 | Magenta |
| 500 dinars | 0.07 | Magenta |
| 1000 dinars | 0.06 | Magenta |
| 2000 dinars | 0.06 | Magenta |
| 5000 dinars | 0.06 | Magenta |

Banknotes with optical density increase higher than values determined in Table 2 are unfit. The NBS keeps reference banknotes showing a soil level derived from these criteria. The densitometric measurements of the reference banknotes are based on the following criteria:

– Standard for density measurements: ISO 5 parts 3 and 4

– Standard for the filters: DIN 16536

– Absolute measurements: standard calibration (white tile)

– Polarisation filter: on

– Aperture: 3 mm

– Illumination: D65/2

- Background: white tile standard calibration

The density increase of a reference banknote is the highest value between the averages of at least four measurement points measured on the front and on the back of the banknote in the unprinted area and without any watermark modulation.

2. Stain

Banknotes with a localised concentration of dirt covering at least 9 mm by 9 mm in the non-printed area or at least 15 mm by 15 mm in the printed area are unfit.

3. Graffiti

At present there is no mandatory requirement to detect graffiti.

4. De-inked note

De-inking of banknotes can occur, e.g. if they have been washed or subjected to aggressive chemical agents. These kinds of unfit banknotes might be detected by image detectors or UV detectors.

5. Tear

Banknotes with tears which are open and not partly or fully covered by the machine’s transport belt(s) are unfit if the size of the tear is greater than indicated below.

Тable 3

Tear

| Direction | Width | Length |
| --- | --- | --- |
| Vertical | 4 mm | 8 mm |
| Horizontal | 4 mm | 15 mm |
| Diagonal | 4 mm | 18 mm |

6. Hole

Banknotes with holes which are not partly or fully covered by the machine’s transport belt(s) are unfit if the area of the hole is greater than 10 mm2.

7. Mutilation

Banknotes with lengths reduced by 6 mm or more or widths reduced by 5 mm or more are unfit. All measurements relate to differences from the nominal lengths and widths of the banknotes.

8. Repair

A repaired banknote is created by joining parts of banknote(s) together, e.g. by using tape or glue. A banknote with tape covering an area greater than 10 mm by 40 mm and which is more than 50µm thick is unfit.

9. Crumples

Crumpled banknotes can normally be identified if their level of reflectance or stiffness is reduced. There is no mandatory requirement.

10. Limpness

As far as possible, banknotes with very little stiffness are sorted as unfit. As limpness normally correlates with soiling, limp banknotes are generally also detected via soil sensors. There is no mandatory requirement.

11. Fold

Folded banknotes, because of their reduced length or width, can be detected by sensors checking the size of the banknotes. In addition, they can be detected by thickness sensors. However, due to technical limitations, only folds fulfilling the criteria laid down for mutilations, i.e. folds leading to a reduction in length greater than 6 mm or a reduction in width greater than 5 mm, can be identified and are unfit.

12. Folded corner

A banknote with a folded corner with an area of more than 130 mm2 and a minimum length of the smaller edge greater than 10 mm is unfit.